COMPUTER NETWORK PROJECT

Secure Home Dashboard

Submitted to:

Dr. Jehad Hamamreh

Prepared by:

| # | Student Name |
| --- | --- |
| 1 | Rahaf Alyan |
| 2 | Tabark Mdena |
| 3 | Leen Akkawi |

An-Najah National University

Faculty of Engineering and Information Technology

Academic Year 2025-2026

# 1. Project Overview

This project presents the design and implementation of a Secure Home Dashboard using a combination of modern networking and virtualization tools. The project demonstrates a complete network topology with NAT configuration, VPN access, and a web-based dashboard accessible remotely over a secure connection.

## 1.1 Project Title

Secure Home Dashboard — Docker + Tailscale + GNS3 NAT Configuration

## 1.2 Objectives

- Design and simulate a network topology using GNS3

- Configure NAT (Network Address Translation) on a Cisco router

- Deploy a web dashboard using Docker and Heimdall

- Enable secure remote access using Tailscale VPN

- Verify full end-to-end connectivity from the simulated network to the internet

# 2. Tools and Technologies

| Tool | Version | Purpose |
| --- | --- | --- |
| GNS3 | 2.2.58.1 | Network simulation and topology design |
| Docker | Latest | Container platform for running Heimdall |
| Heimdall | linuxserver/heimdall | Web-based home dashboard on port 8080 |
| Tailscale | Latest | Secure VPN for remote access |
| Cisco IOS | 12.4 | Router R1 — NAT and routing configuration |

# 3. IP Addressing Table

| Device | IP Address | Notes |
| --- | --- | --- |
| Host Machine (Tailscale) | 100.90.203.89 | VPN access IP |
| NAT1 (GNS3 Built-in) | 192.168.122.1 | Internet gateway |
| R1 — Fa0/0 (NAT side) | 192.168.122.2 | ip nat outside |
| R1 — Fa0/1 (LAN side) | 192.168.10.1 | ip nat inside / Default gateway |
| UbuntuDockerGuest-1 | 192.168.10.2 | Docker host / Heimdall |
| Dashboard URL | localhost:8080 | http://100.90.203.89:8080 (remote) |

# 4. Network Topology

The network topology consists of four layers connected as follows:

Internet (8.8.8.8)  →  NAT1 (192.168.122.1)  →  R1 (192.168.122.2 / 192.168.10.1)  →  Ubuntu (192.168.10.2)

- NAT1 is the GNS3 built-in NAT device that provides internet access to the simulated network

- Router R1 connects the NAT cloud to the internal LAN subnet 192.168.10.0/24

- UbuntuDockerGuest-1 runs Docker and hosts the Heimdall dashboard

- Tailscale VPN provides secure remote access to the dashboard from any location

# 5. Router R1 — Configuration

## 5.1 Interface Configuration

conf t

interface FastEthernet0/0

ip address 192.168.122.2 255.255.255.0

ip nat outside

no shutdown

!

interface FastEthernet0/1

ip address 192.168.10.1 255.255.255.0

ip nat inside

no shutdown

## 5.2 NAT and Routing Configuration

ip route 0.0.0.0 0.0.0.0 192.168.122.1

access-list 1 permit 192.168.10.0 0.0.0.255

ip nat inside source list 1 interface FastEthernet0/0 overload

end

wr

The NAT overload (PAT) configuration allows all devices on the 192.168.10.0/24 subnet to share the single public-facing IP address on Fa0/0 for internet access.

# 6. UbuntuDockerGuest-1 — Setup

## 6.1 Network Configuration

ip addr flush eth0

ip addr add 192.168.10.2/24 dev eth0

ip route add default via 192.168.10.1

## 6.2 Heimdall Dashboard

Heimdall was deployed using Docker with the linuxserver/heimdall image, exposed on port 8080:

docker run -d --name home-dashboard \

-p 8080:80 -p 8443:443 \

linuxserver/heimdall:latest

The dashboard is accessible at:

- Locally: http://localhost:8080

- Remotely via Tailscale VPN: http://100.90.203.89:8080

# 7. Tailscale VPN Configuration

Tailscale was installed on the host machine and connected to a Tailscale account. The assigned VPN IP address is 100.90.203.89. Any device with Tailscale installed and connected to the same account can securely access the Heimdall dashboard without exposing it to the public internet.

- Tailscale uses WireGuard protocol under the hood for encrypted tunneling

- No port forwarding or firewall rules needed on the host router

- Dashboard accessible from any device on the same Tailscale network

# 8. Adding New Devices to the Network

To add a new Ubuntu device to the topology, follow these steps:

## 8.1 In GNS3

- Drag a new UbuntuDockerGuest device onto the canvas

- Connect it to R1 on Fa0/1 (or a new interface if available)

## 8.2 On the New Device

ip addr add 192.168.10.3/24 dev eth0

ip route add default via 192.168.10.1

Each additional device uses the next available IP in the 192.168.10.x range (e.g., .3, .4, .5...)

# 9. Connectivity Verification

| Test | Command | Result |
| --- | --- | --- |
| Router can reach NAT | ping 192.168.122.1 | ✅ Success |
| Ubuntu can reach Router | ping 192.168.10.1 | ✅ Success |
| Internet connectivity | ping 8.8.8.8 | ✅ Success |
| Dashboard accessible | http://localhost:8080 | ✅ Success |
| Remote VPN access | http://100.90.203.89:8080 | ✅ Success |

# 10. Conclusion

This project successfully demonstrates the integration of network simulation, containerization, and VPN technology to build a secure and remotely accessible home dashboard. The complete setup includes:

- A fully functional GNS3 network topology with NAT configuration on Cisco IOS router

- A Docker-based Heimdall dashboard running on a simulated Ubuntu host

- Secure remote access via Tailscale VPN from any device in the world

- Full internet connectivity verified through multiple ping tests

The project showcases practical skills in network design, routing protocols, NAT/PAT configuration, containerization, and modern VPN solutions.